###### ACCREDITATION SCORE SHEET

| Establishment Name: |  |
| --- | --- |
| Address/Phone: |  |
| Owner’s Name: |  |
| Audit date: |  |
| Sales Executive: |  |
| Field Quality Tech: |  |

### SCORING SYSTEM

| 1. | Beer System | 3 standards | 6 |
| --- | --- | --- | --- |
| 2. | Tap Beer Management | 6 standards | 12 |
| 3. | Hygiene | 6 standards | 12 |
| 4. | Beer Quality | 5 standards | 10 |
|  | Total | Points Available | 40 |
|  | Supplements |  |  |
|  | Tanks if used | 1 standard | +2 |
|  | Ice Bank if used | 1 standard | +2 |
| (if ice bank only delete 1.1 & 1.2, total is then 38) |  |  |  |

### CRITERIA

| For each standard give: | 2 points | for satisfactory |
| --- | --- | --- |
|  | 1 point | for minor deficiency |
|  | 0 points | for not met |

0 in any standard in any area = failure

Pass: Must exceed 50% in any area and overall score must exceed 80% (eg, minimum of 32 out of 40 possible).

Where ice banks and/or tanks are used this can affect the total number of points available

The scoring will only be carried out only by a Lion approved assessor.

| TOTAL SCORE ACHIEVED |  |
| --- | --- |

| PASS |  | FAIL |  |
| --- | --- | --- | --- |

(Please tick √ the appropriate box)

AREA 1: Three standards to be achieved. Maximum 2 points per standard (Plus standards for tanks & Icebanks if fitted)

| COMMENTS |  | BEER SYSTEM |  |  |
| --- | --- | --- | --- | --- |
|  |  | Points | 1.1 System Refrigeration: - Condenser unit clean, grills clean and condenser clear of obstruction. - Knowledge of correct operation of cellar, chilled water or ice bank - Coolroom and refrigeration well maintained to ensure that variation of temperature is minimised, eg, insulation, repair to gaps and draft prevention. - Cellar temperature at 0-1oC or correct for ambient cooling system. - Chilled water temperature at -2o to -4oC. |  |
|  |  | Points | 1.2 Cellar Equipment: - Thermometer present – ideally with maximum/minimum. - Cellar cards, instructions and records present. - All coolroom cellar equipment maintained and stored correctly. |  |
|  |  | Points | 1.3 Gas System: - Gas system tidy with no leaks, all gauges working and operator knowledge of correct pressures. - Correct gas type for all beers. |  |
|  |  | Points | 1.4 Tanks (Where in use): - Tanks clean inside and out with no sign of beer stain - Visibly clean sight glass - Door gaskets clean and in good order with no sign of perishing or cuts - CIP cleaning system fitted and used correctly with approved detergent. - Cleaning/filling sheets in place and filled in correctly. |  |
|  |  | Points | 1.5 Ice bank (Where used): - No sign of bacteria in ice bath and inhibitor used. - Icebank refrigeration running correctly. - Icebank cooler grills clean – if fitted. - Stirrer/fan motors all operating. |  |

| Points Achieved |  | (6 Maximum) (8) with tanks or additional ice bank (10) with tanks and ice bank |
| --- | --- | --- |

AREA 2: Six standards to be achieved. Maximum 2 points per standard

| COMMENTS |  | TAP BEER MANAGEMENT AND SAFETY |  |  |
| --- | --- | --- | --- | --- |
|  |  | Points | 2.1 Stock Control: - All beers must be within best by date. - Throughputs for all beers within brewery guidelines. - Evidence of product stock rotation. - Identification of next stock to be used. |  |
|  |  | Points | 2.2 Brand Performance: - Outlet able to give accurate information on throughput of main brand beers. - Knowledge of minimum throughputs of beer. |  |
|  |  | Points | 2.3 Staff Training: - Staff awareness of training requirements including safety, particularly staff who work in cellar. - Staff awareness/knowledge – fresh glass every serve - Staff aware of correct methods of dispensing for all beer types |  |
|  |  | Points | 2.4 Gas System Safety: - Gas cylinders stored correctly, chained or in cupboard. - Presence of gas safety instructions - Knowledge of risks of CO2 and nitrogen gases. |  |
|  |  | Points | 2.5 Chemical Safety - Presence of protective clothing, goggles, gloves, outer clothing, footwear for line cleaning. - Presence of signage on hazardous materials. |  |
|  |  | Points | 2.6 Manual Handling: - Knowledge of safe working practice for moving & stacking kegs. |  |

| Points Achieved |  | (12 Maximum) |  |
| --- | --- | --- | --- |

AREA 3: Four standards to be achieved for each area covered. Maximum 2 points per standard

| COMMENTS |  | HYGIENE |  |
| --- | --- | --- | --- |
|  |  | Points | 3.1 Coolroom Appearance/Condition: - Coolroom walls and roof – clean and free from mould, dirt, beer splashes and exposed insulation damage. - Floor – clean and free from beer residues, mould, debris, cracks and dust - No food or unsealed wood present in coolroom (A small amount of food in sealed containers, unlikely to cause a flavour taint and/or a few crates of Quarts (regularly rotated) may be allowed at the Auditor’s discretion) |
|  |  | Points | 3.2 Cellar Equipment: - Beer lines – no visible yeast deposits on internal surfaces. - Keg / tank couplings, beer lines, fobs, backing boards, ducts, cleaning vessel and other fittings – free from dirt and debris. - Evidence that beer lines are cleaned fortnightly for 0oC cool rooms and weekly for any ambient type cooling system, with approved detergents. |
|  |  | Points | 3.3 Under Bar/Back Bar: - Under bar area clean – free from dust, cobwebs and mould. - Glass storage shelves clean. - Glass storage racks clean. - Back bar area clean. |
|  |  | Points | 3.4 Beer Dispensing: - Dispense towers and drip trays clean. - Taps and shanks clean and made of stainless steel or plastic. |
|  |  | Points | 3.5 Drainage: - Presence of adequate drainage where applicable – drain clean and sump pump working - No smell from drains, evidence of effort to keep clean or attention to alternative means of managing spills. |
|  |  | Points | 3.6 Glassware Presentation: - Visually clean. - Knowledge of glass wash and soaking requirements. - Glass washer, minimum output temperature 77oC for rinse – detergent wash 60oC with approved detergent used. - No evidence of adverse effect on head retention from glasses. |

| Points Achieved |  | (12 Maximum) |  |
| --- | --- | --- | --- |

AREA 4: Five standards to be achieved. Maximum 2 points per standard

| COMMENTS |  | BEER QUALITY |  |  |
| --- | --- | --- | --- | --- |
|  |  | ##### Try all Lion tap beer brands: |  |  |
|  |  | Points | 4.1 Head Presentation: - Head presents at 5mm in unlined glass – good head retention. |  |
|  |  | Points | 4.2 Clarity: - No visual haze or solids |  |
|  |  | Points | 4.3 Flavour/Aroma: - True to type - No off flavours or aromas |  |
|  |  | Points | 4.4 All Glasses Beer Temperature: - Beer temperature between 2 & 4oC in glass - Temperature of beer in 3rd glass (one beer only) |  |
|  |  | Points | 4.5 Glasses: - Glassware is suitable for beer - Glass washer machine clean and free from waste build up. - If glassware branded use correct branded glass. |  |

| Points Achieved |  | (10 Maximum) |  |
| --- | --- | --- | --- |